# HARMONEX Lab Notebook – Hardware Development

## Date: September 5, 2025

Objectives:
Mechanical architecture definition

Record of what was done:
Defined overall mechanical architecture of the elbow module, including hybrid actuation layout with BLDC-driven planetary gearbox and servo-driven linkage. Established modular skeleton concept using PA-CF to balance stiffness and weight.

## Date: September 12, 2025

Objectives:
Planetary gearbox design

Record of what was done:
Designed multi-stage planetary gearbox for BLDC drive. Selected module-1 spur gears with 20-tooth sun and 60-tooth ring gears to achieve approximately 64:1 reduction across three stages. Verified torque capacity using printed PLA prototypes.

## Date: September 18, 2025

Objectives:
Clutch mechanism design

Record of what was done:
Designed dog-clutch geometry to couple BLDC gearbox output to elbow joint. Iterated tooth profile and engagement chamfers to improve alignment tolerance and reduce jamming during engagement.

## Date: September 25, 2025

Objectives:
Skeleton and shell prototyping

Record of what was done:
Printed first-generation elbow skeleton and shell components in PLA. Evaluated fit, range of motion, and comfort. Identified interference regions near extreme elbow angles.

## Date: October 3, 2025

Objectives:
Material transition to carbon-fiber filaments

Record of what was done:
Migrated structural components to PA6-CF and PETG-CF. Tuned print parameters to mitigate warping and shrinkage. Reprinted load-bearing beams and joint housings.

## Date: October 10, 2025

Objectives:
Servo drivetrain development

Record of what was done:
Designed spur, bevel, and rack-and-pinion linkage for MG90S servo. Integrated linkage inside elbow shell to preserve external profile and reduce snag hazards.

## Date: October 17, 2025

Objectives:
Clutch alignment sensing integration

Record of what was done:
Added rotary potentiometer to measure angular misalignment between clutch halves. Updated mechanical CAD to include sensor mount and wiring channels.

## Date: October 24, 2025

Objectives:
Thermal and airflow considerations

Record of what was done:
Evaluated heat generation from buck converters and BLDC ESC. Designed airflow ducts and passive fins integrated into skeleton geometry to guide forced convection.

## Date: October 31, 2025

Objectives:
Power PCB assembly

Record of what was done:
Assembled power PCB including 5V→3.3V buck, 3.3V→8V boost, 8V→–8V charge pump, and INA240 current sensing. Performed continuity checks and initial power-on testing.

## Date: November 6, 2025

Objectives:
Current sensing validation

Record of what was done:
Validated INA240 current sensing using bench power supply and resistive loads. Confirmed linear response and noise immunity during simulated motor transients.

## Date: November 13, 2025

Objectives:
Mechanical load testing

Record of what was done:
Performed static load tests with up to 10 kg at 90° elbow angle. Verified no visible deformation or clutch slip under sustained loading.

## Date: November 20, 2025

Objectives:
Clutch strength testing

Record of what was done:
Printed PA-CF dog-clutch tensile specimens. Conducted pull tests to estimate ultimate tensile strength and observed failure modes.

## Date: November 27, 2025

Objectives:
Final mechanical integration

Record of what was done:
Integrated BLDC module, servo drivetrain, clutches, skeleton, and power electronics into final elbow assembly. Verified cable routing, strain relief, and serviceability.

## Date: December 1, 2025

Objectives:
Pre-demo inspection and fixes

Record of what was done:
Inspected all fasteners, electrical connectors, and printed parts before final demo. Replaced moisture-affected PA-CF components and re-torqued critical joints.