Request for Approval

ECE 445, 25SP

Team Members:

Jitao Li, jitaoli2

Teng Hou, tenghou2

Yikai Cao, yikaic2

Wenkang Li, wenkang2

- Problem

There are various commercially available robotic dog platforms, yet no “fetching” skill is shown. One reason is the lack of integration of a manipulator with the dog. To be compatible with the robot dog, the robot arm needs to be lightweight, accurate, and robust. The integrated system will be able to perform simple tasks such as fetching, with the help of visual feedback. Such a manipulator requires a new design, good coordination of its components, along with a dedicated controller.

- Solution Overview

Our solution for building such a robotic platform begins with building a 6-DoF robot arm from 3D printed components and actuators. Our robot arm is light enough to be mounted on the robot dog. The arm can move and precisely grasp an everyday object, with a workspace radius of roughly 0.5m. Next, we integrate the arm with the robot dog. By combining control signals of the dog and the arm and data from visual sensors, the dog can identify a certain object in its surroundings and successfully fetch it. Additionally, a controller will be built for our robot arm, and will be integrated with the on-board controller of the robot dog to achieve all-in-one control of the entire system. A human operator can remote control it to move and fetch an object.

- Solution Components

Robot arm:

3D printed links of the robot arm

Actuators for the joints of the robot arm

A gripper as an end effector for grasping

3D printed base for mounting the robot arm on the robot dog

Self-built controller for the robot arm

Embedded control system:

Microcontroller (e.g., STM32 or similar) for arm control

Motor drivers for actuator control

IMU sensor for arm position feedback

Power management circuit for voltage regulation

PCB design for integrating electronic components

Communication interface (UART/I2C) for dog-arm coordination

Vision:

Camera module for object detection

Image processing unit

Object recognition and tracking algorithms

Robot dog:

A commercial robot dog

- Criteria for success

The robot arm must be assembled and each joint must rotate.

The end point of the robot arm must reach a designated point in space precisely without shivering.

The integrated robot dog must identify the object to fetch with on-board and additional cameras.

The robot dog mush approach the given object and the robot arm must grasp it with its gripper.