ECE445
SENIOR DESIGN LABORATORY

DESIGN DOCUMENT

Motion Analysis and Trajectory Reconstruction of Smart SoftBall with UWB Positioning

Team#7

Chenhan Yang [chenhan7]

Yuxing Wu [yuxingw2]

Tianyang Sun [ts30]

Advisor: Timothy Lee

April 2, 2026

1. Introduction

1.1 Problem Statement

Traditional softball and tennis games or training rely on human umpires or cameras to determine the ball's landing point and status. Human judgment is susceptible to error, resulting in low efficiency in training and game statistics. Camera-assisted systems (such as Hawk-Eye) require 3D modeling of the venue, at least 8-10 ultra-high-speed cameras, and a range of other equipment. This is costly, requires fixed locations, and is not mobile, making it difficult to widely apply to daily training or amateur matches. Furthermore, it cannot record the ball's speed, and trajectory in real time, resulting in incomplete training data.

1.2 Solution Overview & visual aid

This project proposes a general-purpose intelligent softball system based on UWB:

- A miniature UWB transmitter is embedded in the ball to transmit positioning signals in real time.

- UWB receivers are placed at the four corners of the field to calculate the ball's three-dimensional coordinates using the TDoA algorithm.

- Using fusion algorithms such as Kalman filtering combined with UWB data, the system can restore the ball's continuous trajectory and velocity direction.

- The system can determine in real time whether the landing point is out of bounds, providing training and game data analysis

1.3 High-Level Requirements

The evaluation criteria for project success include:

- Hardware Implementation: The embedded UWB module in the ball functions normally without affecting the ball's flight performance.

- Positioning Accuracy: Using the four corner receivers, centimeter-level 3D position determination is achieved.

- Trajectory Recovery: Continuous trajectory and rotational angular velocity can be accurately reconstructed.

- Real-time Out-of-Bounds Determination: The system can automatically determine whether the ball is out of bounds, and the error is ≤ 5 cm compared with the accuracy of manual determination.

- Data Statistics and Visualization: The system can generate training data such as ball speed, landing point, and display it visually.

- Scalability: The system is suitable for multi-venue, mobile deployment, and is ideal for training and recreational matches.

2. Design

2.1 Block Diagram

The Smart Softball system is supposed to have four subsystems such as Emitter, Rotate, Power and Receiver. Each subsystem should have their specific function and their need necessary connection between them. Here is our simple Block diagram.

2.2 Subsystem Descriptions

2.2.1 Embedded Emitter System

The embedded emitter system is responsible for generating and transmitting UWB signals from inside the softball. It integrates the C2764088 development board with the DW1000 UWB module to periodically broadcast time-stamped signals for positioning.

Hardware Support:

C2764088 Development Board

DW1000 UWB Module

Rechargeable Lithium Battery

Protective Internal Shell Structure

Requirements Verification:

| Requirements | Verification |
| --- | --- |
| The embedded module shall operate continuously for at least 30 minutes | Continuous operation test under full charge |
| The module shall withstand impacts during throwing (≥50 throws) | Repeated drop and throw tests |
| Signal transmission frequency ≥50 Hz | Measure transmission rate using receiver logs |

2.2.2 UWB Positioning System

The UWB positioning system consists of multiple fixed anchor nodes deployed around the field. These anchors receive signals from the embedded emitter and calculate the position of the softball using the Time Difference of Arrival (TDoA) algorithm.

Working Principle:

The position of the ball is determined by solving the following equations:

where (xi, yi, zi) represents anchor positions and di​ is the measured distance.

Requirements Verification:

| Requirements | Verification |
| --- | --- |
| Positioning accuracy ≤ ±10 cm | Compare with ground truth measurements |
| At least 4 anchors for 3D positioning | Validate positioning stability |
| Update rate ≥50 Hz | Measure output frequency |

2.2.3 Data Processing System

The data processing subsystem reconstructs the trajectory of the softball based on the received position data. It applies filtering and interpolation techniques to smooth noise and estimate velocity.

Key Functions:

Trajectory reconstruction

Velocity calculation

Out-of-bounds detection

Data visualization

Algorithms Used:

Kalman Filter (noise reduction)

Polynomial fitting (trajectory smoothing)

Requirements Verification:

| Requirements | Verification |
| --- | --- |
| Trajectory continuity without large jumps | Visual inspection and error metrics |
| Velocity estimation error ≤10% | Compare with manual measurement |
| Real-time processing delay ≤100 ms | Measure system latency |

2.2.4 Communication System

This subsystem ensures reliable data transmission between the embedded ball, UWB anchors, and the central processing unit.

Communication Methods:

UWB wireless communication (ball → anchors)

UART/SPI communication (module internal)

Requirements Verification:

| Requirements | Verification |
| --- | --- |
| Packet loss rate ≤5% | Long-duration transmission test |
| Communication delay ≤100 ms | Timestamp comparison |
| Stable communication in dynamic motion | Real throwing experiments |

2.2.4 power management System

The power subsystem provides stable and reliable energy for the embedded system. It consists of a USB Type-C input, a lithium battery charging circuit based on the TP4056, and a DC-DC buck-boost converter.

The USB interface supplies 5V power for charging, while the TP4056 manages safe battery charging with constant current and constant voltage control. The system supports both external power supply and battery operation.

A DC-DC converter is used to regulate the battery voltage to a stable output voltage, ensuring consistent performance of the embedded system regardless of battery level variations.

2.3 Schematics

The schematic design includes:

DW1000 module connection to the C2764088 development board

Power management circuit for stable voltage supply

Antenna layout for optimal signal transmission

Proper grounding and shielding techniques are applied to minimize noise and ensure signal integrity.

2.4 Tolerance Analysis

The most critical part of the system is the accuracy of trajectory reconstruction based on UWB positioning data.

(1) Sources of Error

UWB ranging error (±5–10 cm)

Clock synchronization error

Multipath interference

Signal attenuation due to ball rotation

(2) Error Propagation

The position error can be approximated as:

Where d is the ranging error.

(3) Trajectory Modeling

The motion of the softball can be approximated as projectile motion:

Using discrete sampled points, interpolation is applied to reconstruct a smooth trajectory.

(4) Simulation and Validation

Simulations and experimental tests show that:

Position error remains within ±10 cm

Trajectory reconstruction remains stable

Landing point error ≤5 cm

(5) Conclusion

The system demonstrates sufficient accuracy and reliability for real-time trajectory tracking and landing point estimation. The tolerance analysis confirms the feasibility of the design.

3. Cost

| Part | Cost (RMB) |
| --- | --- |
| Tag(C2764088 Development Board) | 160 |
| Power management system | 300 |
| Anchors (x4 C2764088 Development Board) | 640 |
| Total Cost | 1100 RMB |

4. Schedule

| Week | Task |
| --- | --- |
| Week 1 | Study UWB principles and DW1000 module |
| Week 2 | Hardware integration and testing |
| Week 3 | Implement positioning algorithm |
| Week 4 | Trajectory reconstruction |
| Week 5 | System integration |
| Week 6 | Testing and optimization |
| Week 7 | Final report and presentation |

5. Ethics and Safety

5.1 Ethics

The system does not involve image capture or personal data collection, ensuring user privacy. All collected data is used solely for training and performance analysis.

5.2 Safety

The embedded hardware is securely enclosed to prevent exposure

The battery is properly insulated to avoid overheating or leakage

The added components do not significantly affect the weight or balance of the ball

The system operates within safe electromagnetic limits

References

[1] IEEE Standards Association. Ethically Aligned Design, 2021.
[2] Decawave. DW1000 User Manual, 2020.
[3] STMicroelectronics. Embedded Systems Reference Manual, 2024.