# ECE 486 Extra Credit

Integral Windup and Back-Calculation Anti-Windup

Student Assignment and Report Template

| Student information | Entry |
| --- | --- |
| Name | [Enter name] |
| NetID | [Enter NetID] |
| Section | [Enter section] |
| Lab TA | [Enter TA] |
| RWP number | [Enter number, if applicable] |
| Date | [Enter date] |

# How to Complete This Extra-Credit Assignment

This extra-credit assignment is completed at home and submitted electronically. Submit a freestanding report that contains your derivations, MATLAB calculations, Simulink diagrams, logged data, and MATLAB-generated plots. All equations, code, plots, and explanations must be your own work.

- Use the exact controller convention stated in Section 2: the integral gain Ki is placed after the integrator.

- Analyze the saturation as a four-phase switched system. A saturation block is nonlinear and does not have one transfer function that is valid in every phase.

- At the start of each phase, introduce a local time variable τ=0 and carry the final states from the previous phase into the new phase.

- Show all calculations needed to support every reported result.

- Use MATLAB to solve transcendental crossing equations and to verify analytical results. Use the fill-in root-finding scaffold in Appendix A and include the completed code in your submitted MATLAB .m file.

- Build both Simulink systems and compare them using identical parameters.

- The delayed-enable windup scenario is simulation only. Do not intentionally reproduce it on the physical RWP motor.

# Grading Summary

| Section | Topic | Points |
| --- | --- | --- |
| 3 | Signal definitions and saturation interpretation | 2 |
| 4 | Identification of the four phases | 3 |
| 5 | Phase 1 analysis | 4 |
| 6 | Phase 2 with anti-windup | 6 |
| 7 | Phase 2 without anti-windup | 3.5 |
| 8 | Phase 3 with anti-windup | 7.5 |
| 9 | Phase 3 without anti-windup | 4.5 |
| 10 | Phase 4 with anti-windup | 7.5 |
| 11 | Phase 4 without anti-windup | 4 |
| 12 | Simulink model construction | 2.5 |
| 13 | Required plots and switching markers | 1.5 |
| 14 | Analytical and simulation comparison table | 2 |
| 15 | Physical interpretation and comparison | 1.5 |
| 16 | Conclusion | 0.5 |
| Total |  | 50 |

# 1. Scenario and Objective

A student wishes to regulate the reaction-wheel velocity to a constant reference. The student starts the PI controller but forgets to enable the motor amplifier. The motor remains stationary, but the controller continues calculating the error and updating its integral state. After several seconds, the student enables the motor. The motor immediately receives its maximum allowed command, accelerates aggressively, overshoots the requested velocity, takes a long time to settle, and overheats.

The student is confused because a Saturation block limited the motor command to ±10. The student is then advised to add feedback anti-windup using the difference between the saturated and unsaturated controller commands.

Your objectives are to:

- explain why saturation alone does not prevent integral windup;

- derive Us, Es, and Ys in each of four operating phases;

- determine phase-transition times, steady values, time constants, saturation-release times, peak output, percent overshoot, and settling time;

- compare the controller with and without saturation feedback in Simulink.

# 2. Given System, Parameters, and Controller Convention

Use the ideal reaction-wheel velocity model

Ps=brs,  yt=bruat.

The reference is

rt=r0,  Rs=r0s.

Use the following values in all calculations and simulations.

| Parameter | Value |
| --- | --- |
| Reference r0 | 100 rad/s |
| Plant gain br | 198 |
| Proportional gain Kp | 0.095 |
| Integral gain Ki | 0.030 |
| Anti-windup gain Ka | 40 |
| Positive limit Umax | 10 |
| Negative limit Umin | −10 |
| Motor-enable time ton | 15.000 s |
| Initial conditions | y0=0, xI0=0 |

The controller convention used throughout this assignment is

ut=Kpet+KixIt

xIt=et+Kausatt−ut.

Thus, the signal order is

e+Kausat−u → 1s → Ki.

The gain Ki is after the integrator. Do not use the alternative convention in which Ki multiplies the error before integration.

Back-calculation anti-windup block diagram with the integral gain Ki after the integrator, a saturation block, a motor-enable switch, and the plant.

The saturation and motor-enable relations are

usatt=Umax,ut>Umax,ut,Umin≤ut≤Umax,Umin,ut<Umin,

uat=0,t<ton,usatt,t≥ton.

For the comparison without anti-windup, keep the same Saturation block but set

Ka=0,  xI=e.

# 3. Signal Definitions and Saturation Interpretation — 2 points

Explain the physical and mathematical difference among:

- ut: the unsaturated internal PI-controller command;

- usatt: the output of the Saturation block;

- uat: the command actually applied to the motor.

Explain why usat≤Umax does not imply u≤Umax. Explain why limiting command magnitude does not necessarily limit the duration for which the motor remains at Umax.

Hint: The saturation block is memoryless, but the integrator has memory.

Student response:

[Enter response.]

# 4. Identification of the Four Phases — 3 points

Identify the four phases and state the conditions on ua, usat, and u that define each phase.

- Phase 1: motor disabled and saturation inactive.

- Phase 2: motor disabled and positive saturation active.

- Phase 3: motor enabled and positive saturation remains active.

- Phase 4: motor enabled and saturation is inactive again.

For each phase, state whether yt, et, and ut are constant, increasing, decreasing, or governed by closed-loop dynamics.

Hint: The boundary between Phases 3 and 4 occurs when ut crosses Umax while moving downward. It does not occur when the output crosses the reference.

| Phase | Motor state | Saturation state | Defining equations and expected behavior |
| --- | --- | --- | --- |
| 1 | [Enter] | [Enter] | [Enter] |
| 2 | [Enter] | [Enter] | [Enter] |
| 3 | [Enter] | [Enter] | [Enter] |
| 4 | [Enter] | [Enter] | [Enter] |

# 5. Phase 1: Motor Disabled, Saturation Inactive — 4 points

During Phase 1,

ua=0,  usat=u<Umax.

## 5.1 Output and error

Derive Y1s and E1s.

## 5.2 Controller command

Derive U1s and u1t. Determine:

- the initial command u₁(0);

- the slope of u1t;

- whether a finite Phase 1 steady value exists;

- the first saturation-crossing time t1 satisfying u1t1=Umax.

Hints:

- The anti-windup feedback is zero because usat=u.

- The output is zero, so the step error is constant.

- Check whether Kpr0<Umax before using a positive-duration Phase 1 formula.

| Phase 1 quantity | Analytical result | Numerical value |
| --- | --- | --- |
| Y1s | [Enter] | [Enter] |
| E1s | [Enter] | [Enter] |
| U1s | [Enter] | [Enter] |
| u1t | [Enter] | [Enter] |
| Initial command | [Enter] | [Enter] |
| First saturation time t1 | [Enter] | [Enter] |

# 6. Phase 2 with Anti-Windup — 6 points

Phase 2 begins at t=t1. Define local time

τ=t−t1.

At τ=0,

u20=Umax.

During Phase 2,

ua=0,  usat=Umax,  e=r0.

## 6.1 Laplace-domain expressions

Derive Y2s, E2s, and U2s. Your U2s expression must include:

- the contribution from E2s;

- the input Umax/s;

- the inherited integral state xI0 from Phase 1.

## 6.2 Steady values and anti-windup dynamics

Determine:

- u2,ss;

- xI,2,ss;

- the Phase 2 pole;

- the anti-windup time constant τaw;

- the approximate 2% settling time ts,aw.

## 6.3 State at motor enable

Determine the Phase 2 duration

Δt2=ton−t1

and calculate

uonAW=u2Δt2,  xI,onAW=xIΔt2.

Explain whether the internal anti-windup dynamics have approximately settled before the motor is enabled.

Hints:

- Define a=KiKa.

- A useful controller equation is u+au=Kie+aUmax because e is constant in Phase 2.

- The Phase 2 settling time concerns u and xI, not y; the motor is still disabled.

| Phase 2 anti-windup quantity | Analytical result | Numerical value |
| --- | --- | --- |
| u2,ss | [Enter] | [Enter] |
| xI,2,ss | [Enter] | [Enter] |
| Pole | [Enter] | [Enter] |
| τaw | [Enter] | [Enter] |
| Approximate ts,aw | [Enter] | [Enter] |
| uonAW | [Enter] | [Enter] |
| xI,onAW | [Enter] | [Enter] |

# 7. Phase 2 without Anti-Windup — 3.5 points

Repeat Phase 2 with

Ka=0.

Derive XI,2noAWs, U2noAWs, and u2noAWτ. Determine:

- whether a finite steady value exists;

- whether a finite time constant or settling time exists;

- uonnoAW;

- xI,onnoAW.

Compare the motor-enable states with and without anti-windup.

Hint: The Saturation block limits usat but does not appear in the integrator differential equation when Ka=0.

| Quantity at motor enable | With anti-windup | Without anti-windup |
| --- | --- | --- |
| Internal command uon | [Enter] | [Enter] |
| Integral state xI,on | [Enter] | [Enter] |

# 8. Phase 3 with Anti-Windup — 7.5 points

Phase 3 begins when the motor is enabled at t=ton. Define

τ=t−ton.

Use the inherited conditions

y30=0,  e30=r0,  u30=uonAW,  xI0=xI,onAW.

During Phase 3,

ua=usat=Umax.

## 8.1 Output Y3s

Derive Y3s and y3τ.

## 8.2 Error E3s

Derive E3s and e3τ. Determine the time at which the output would cross the reference if positive saturation remained active.

## 8.3 Internal command U3s

Derive U3s including the inherited integral state. Obtain the corresponding time-domain expression u3τ.

## 8.4 Positive-saturation release

Determine the first positive root τ3AW satisfying

u3τ3AW=Umax

while u3 is decreasing. MATLAB numerical root finding may be used.

Calculate

yrAW=y3τ3AW,  erAW=e3τ3AW,  xI,rAW.

State whether the output has crossed the reference before positive saturation is released.

Hints:

- Define B=brUmax and a=KiKa.

- During Phase 3, y is a ramp and e is a decreasing ramp.

- The equation for u3 contains both a ramp term and an exponential term, so the release equation is generally transcendental.

MATLAB requirement: use the fill-in fzero scaffold in Appendix A for the positive-saturation release root and include the completed code in your submitted .m file.

| Phase 3 anti-windup quantity | Result |
| --- | --- |
| Y3s | [Enter] |
| E3s | [Enter] |
| U3s | [Enter] |
| u3τ | [Enter] |
| Reference-crossing time if saturation continued | [Enter] |
| Release time τ3AW | [Enter] |
| Output at release yrAW | [Enter] |
| Error at release erAW | [Enter] |

# 9. Phase 3 without Anti-Windup — 4.5 points

Repeat Phase 3 with Ka=0 and the inherited no-anti-windup state from Section 7.

- Compare Y3AWs with Y3noAWs while both systems remain positively saturated.

- Compare E3AWs with E3noAWs during the same interval.

- Derive U3noAWs and u3noAWτ.

- Determine τ3noAW from u3noAWτ=Umax.

- Determine yrnoAW and ernoAW.

- Explain why the two output responses initially look identical even though they leave saturation at different times.

Hint: Without anti-windup, xI is the integral of a ramp error, so u3noAWτ is quadratic.

MATLAB requirement: repeat the Appendix A root-finding procedure for the no-anti-windup release condition and include the completed code in the same .m file.

# 10. Phase 4 with Anti-Windup — 7.5 points

Phase 4 begins when u crosses Umax on the way down. Define local time τ=0 at that instant. The inherited conditions are

y40=yrAW,  e40=erAW,  u40=Umax.

Now

usat=u,  ua=u,

so the anti-windup correction is zero and the ordinary PI controller is restored.

## 10.1 Zero-initial-condition transfer functions

Derive

Y4sRs,  E4sRs,  U4sRs.

Identify the characteristic polynomial, poles, ωn, and ζ. State whether ωd is real.

## 10.2 Actual response with inherited states

Derive the actual E4s, U4s, and Y4s using e40=erAW and u40=Umax.

## 10.3 Peak output and overshoot

Determine the first time τpAW satisfying

u4τpAW=0.

Calculate

ymaxAW=y4τpAW,  MpAW=ymaxAW−r0r0×100%.

Explain why the output peak normally occurs after positive saturation has been released.

## 10.4 Settling time

Calculate:

- the dominant-pole estimate for the Phase 4 settling time;

- the actual Phase 4 2% settling time using the inherited-state response;

- the overall 2% settling time measured from t=0.

Hints:

- For a constant reference, e=−bru.

- The Phase 4 characteristic polynomial is s2+brKps+brKi.

- The output is at a maximum when y=bru=0, not when u first falls below Umax.

# 11. Phase 4 without Anti-Windup — 4 points

Repeat the actual Phase 4 analysis using the no-anti-windup release conditions

e40=ernoAW,  u40=Umax.

Determine:

- E4noAWs, U4noAWs, and Y4noAWs;

- τpnoAW;

- ymaxnoAW;

- MpnoAW;

- the actual Phase 4 and overall 2% settling times.

Explain why both systems have the same Phase 4 poles but different overshoot and settling time.

Hint: The dynamics are the same, but the systems enter Phase 4 with different initial errors and integral states.

# 12. Simulink Model Construction — 2.5 points

Create two Simulink models, or one model with a clearly labeled switch, for:

- Model A: saturation without anti-windup, Ka=0;

- Model B: saturation with back-calculation anti-windup, Ka=40.

Both models must use identical values of Kp, Ki, r0, Umax, br, and ton. The only control-structure difference is the feedback term Kausat−u.

Requirements:

- place Ki after the integrator;

- include the Saturation block before the motor-enable switch;

- force the actual motor input to zero before ton;

- log all required signals to MATLAB;

- use a sufficiently small simulation step to resolve the short Phase 3 saturation-release interval. A fixed step of 5×10−4 s or smaller is recommended. Use a simulation stop time of 40 s.

Place complete Simulink diagrams here or in an appendix and reference them here.

# 13. Required Plots and Switching Markers — 1.5 points

Generate plots using MATLAB code from logged Simulink data. Do not submit manually drawn plots.

Plot, for both models:

rt, yt, et, ut, usatt, uat, xIt.

Clearly mark:

- t1: first positive saturation crossing;

- ton: motor enable;

- t3: positive-saturation release;

- tr: first reference crossing;

- tp: peak-output time;

- ts: overall 2% settling time.

# 14. Analytical and Simulink Comparison — 2 points

Complete the table. Use “not applicable” where no finite steady value or settling time exists.

| Quantity | No anti-windup analytical | No anti-windup Simulink | Anti-windup analytical | Anti-windup Simulink |
| --- | --- | --- | --- | --- |
| First saturation time t1 |  |  |  |  |
| uton |  |  |  |  |
| Maximum integral state |  |  |  |  |
| Phase 2 steady u |  |  |  |  |
| Phase 2 time constant |  |  |  |  |
| Phase 2 settling time |  |  |  |  |
| First reference-crossing time tr |  |  |  |  |
| Positive-saturation release time |  |  |  |  |
| Time at +Umax after motor enable |  |  |  |  |
| Output at saturation release |  |  |  |  |
| Peak-output time |  |  |  |  |
| Maximum output |  |  |  |  |
| Percent overshoot |  |  |  |  |
| Overall 2% settling time |  |  |  |  |
| Final steady-state error |  |  |  |  |
| Final steady controller command |  |  |  |  |

# 15. Physical Interpretation and Comparison — 1.5 points

Provide a thorough explanation of the following.

## 15.1 Why saturation alone failed

Explain why the saturation block limited the applied command but did not prevent the integral state from growing.

## 15.2 Why the motor stayed at maximum command

Explain why the motor continued receiving Umax after it was enabled, including after the output approached or crossed the reference in the no-anti-windup case.

## 15.3 Why prolonged saturation caused overheating

Distinguish excessive command magnitude from a limited command applied for an excessive duration.

## 15.4 Why anti-windup helped

Explain how Kausat−u bounded the internal command during Phase 2 and shortened Phase 3.

## 15.5 Why the outputs initially looked identical

Explain why both outputs initially had the same slope after motor enable while both actuators were saturated.

## 15.6 Why the peak did not occur at saturation release

Use y=bru to identify the actual peak condition.

## 15.7 Final steady-state behavior

Explain why both ideal models eventually have zero steady-state error even though their transient responses differ greatly.

# 16. Conclusion — 0.5 points

Use your analytical and Simulink results to evaluate the statement:

A saturation block protects the actuator from excessive command magnitude, while anti-windup protects the controller from accumulating an excessive internal command.

Your conclusion must cite at least four numerical comparisons from your results.

# 17. Submission, Academic Integrity, and Safety

Submit:

- the completed report in PDF or DOCX format;

- the Simulink model or models;

- the complete MATLAB analysis and plotting .m script, including both completed Phase 3 root-finding calculations;

- the logged simulation data used to generate the plots.

The intentional delayed-enable windup test must remain simulation-only. The physical RWP motor must not be intentionally operated using the windup scenario.

# Appendix A. MATLAB Root-Finding Scaffold

Use the following fill-in scaffold for the two Phase 3 positive-saturation release calculations. Insert your derived expression for the internal controller command, choose a bracket containing the first positive root, and verify that the selected root is a downward crossing. Include the completed code in your submitted MATLAB .m file.

## A.1 Phase 3 with anti-windup

% Phase 3 positive-saturation release - anti-windup

Umax = 10;

t_on = 15.000;

u3 = @(tau) ... % ENTER your derived u_3(tau) expression

F = @(tau) u3(tau) - Umax;

tauL = ...;     % ENTER a left bracket endpoint

tauR = ...;     % ENTER a right bracket endpoint

assert(F(tauL)*F(tauR) <= 0, ...

'Selected interval does not bracket a root.');

tau_rel = fzero(F,[tauL tauR]);

t_rel = t_on + tau_rel;

h = 1e-6;

du_dtau = (u3(tau_rel+h)-u3(tau_rel-h))/(2*h);

assert(tau_rel > 0 && du_dtau < 0, ...

'Root must be the first positive downward crossing.');

fprintf('AW local release time  = %.9f s\n',tau_rel);

fprintf('AW global release time = %.9f s\n',t_rel);

fprintf('AW command at release  = %.9f\n',u3(tau_rel));

fprintf('AW slope at release    = %.9f (must be < 0)\n',du_dtau);

## A.2 Phase 3 without anti-windup

% Phase 3 positive-saturation release - no anti-windup

Umax = 10;

t_on = 15.000;

u3 = @(tau) ... % ENTER your derived no-AW u_3(tau) expression

F = @(tau) u3(tau) - Umax;

tauL = ...;     % ENTER a left bracket endpoint

tauR = ...;     % ENTER a right bracket endpoint

assert(F(tauL)*F(tauR) <= 0, ...

'Selected interval does not bracket a root.');

tau_rel = fzero(F,[tauL tauR]);

t_rel = t_on + tau_rel;

h = 1e-6;

du_dtau = (u3(tau_rel+h)-u3(tau_rel-h))/(2*h);

assert(tau_rel > 0 && du_dtau < 0, ...

'Root must be the first positive downward crossing.');

fprintf('No-AW local release time  = %.9f s\n',tau_rel);

fprintf('No-AW global release time = %.9f s\n',t_rel);

fprintf('No-AW command at release  = %.9f\n',u3(tau_rel));

fprintf('No-AW slope at release    = %.9f (must be < 0)\n',du_dtau);

The submitted MATLAB script must contain both completed root calculations and must reproduce the numerical release times used in the report. Do not replace the root calculation with a manually typed release time.

# Detailed 50-Point Rubric

| Section | Criterion | Points |
| --- | --- | --- |
| 3 | Correct distinction among u, usat, and ua; correct memory explanation | 2 |
| 4 | Four correct phase definitions and qualitative behavior | 3 |
| 5 | Correct Phase 1 transforms, time response, and first crossing | 4 |
| 6 | Correct Phase 2 anti-windup expression, steady values, pole, time constant, and motor-enable state | 6 |
| 7 | Correct no-anti-windup buildup and motor-enable comparison | 3.5 |
| 8 | Correct Phase 3 Y, E, and U expressions; numerical release condition and release state | 7.5 |
| 9 | Correct no-anti-windup Phase 3 command, release time, and comparison | 4.5 |
| 10 | Correct Phase 4 transfer functions, inherited-state response, peak, overshoot, and settling | 7.5 |
| 11 | Correct no-anti-windup Phase 4 response and comparison | 4 |
| 12 | Correct and comparable Simulink implementations | 2.5 |
| 13 | Complete MATLAB-generated plots with switching markers | 1.5 |
| 14 | Complete analytical-versus-Simulink table | 2 |
| 15 | Accurate physical interpretation | 1.5 |
| 16 | Evidence-based conclusion | 0.5 |
| Total |  | 50 |