# ECE 486 Final Project

The Reaction Wheel Pendulum

Numbered Report Questions and Results Template

Name: [Enter name]

Project partner: [Enter partner or None]

Lab TA: [Enter TA]

Section: [Enter section]

RWP number: [Enter number painted on base]

Date: [Enter date]

# How to Use This Report Template

This document is completed after the laboratory work and is submitted as one formal report per group. Use the numbered tables and prompts to organize required evidence, but write the report as a coherent document rather than as isolated answers. The posted Spring 2026 report guidelines require Times New Roman 11-point student-entered report text and page numbers, and indicate that the necessary material should normally be stated concisely in about 5–7 pages, excluding the cover page.

☐ Show the calculations and design choices needed to support every reported result.

☐ Complete every table and refer to it explicitly in the corresponding numbered discussion.

☐ Keep the required closed-loop stability proof and observer-convergence proof in the relevant report sections. Place MATLAB code, complete Simulink diagrams, and supporting full-size plots in appendices and cite them from the main text.

☐ Write complete explanatory paragraphs with transitions between sections. Label or caption every equation, figure, and table that is referenced in the report.

☐ Include the complete Simulink diagram printouts in the report appendix and reference them from the corresponding discussion.

☐ The numbered questions are not separate point categories by themselves; they supply the calculations, data, simulations, and experimental evidence used in the 130-point report rubric in Table C2.

# Numbered Report Questions, Required Tables, and Discussion

## Report Introduction — 10 points

Introduce the Reaction Wheel Pendulum and give the system information required by the posted final-project report rubric.

Identify the sensors used and state how many are used. (3 points)

Identify the actuator used and state how many are used. (3 points)

Identify the relevant equilibrium positions and state which are stable and unstable configurations. (3 points)

State the program/environment used to implement the controllers on the RWP. (1 point)

Introduction:

## 1-1: Kinetic and Potential Energy

Present the RWP energy model and use it to establish the generalized coordinates used in the report.

- Enter the number of degrees of freedom and identify the generalized coordinates and their time derivatives.

- Derive the total kinetic energy by adding the pendulum and reaction-wheel contributions.

- Derive the gravitational potential energy using the selected angle convention.

- Define every mass, length, and inertia symbol and explain the physical meaning of each term.

Degrees of freedom: [Enter]

Generalized coordinates: θp = [Enter], θr = [Enter]

Kinetic energy: [Enter complete derivation]

Potential energy: [Enter complete derivation]

Report discussion: interpret the energy model and relate it to the RWP coordinates.

## 1-2: Lagrange Equations and Equations of Motion

Derive the nonlinear equations of motion from the energy expressions in Question 1-1.

- Write the Lagrangian and the Lagrange equation for each generalized coordinate.

- Show the generalized motor torques and their equal-and-opposite signs on the pendulum and rotor.

- Present the final coupled equations in the grouped-parameter form used later in the project.

- State the sign convention and define all grouped parameters.

Derivation for the pendulum coordinate:

Derivation for the rotor coordinate:

Final equations of motion and parameter definitions:

## 2-1: Proportional Velocity Control

Report the proportional-controller design and compare the calculated frictionless response with simulation.

- Complete Table A1 using the selected gain, predicted pole, predicted rise time, and simulated response.

- Show the closed-loop transfer function and the calculation used to choose the proportional gain.

- Compare calculated and simulated rise time and steady-state error.

- Explain why the ideal frictionless result does not fully predict the physical motor response.

| Design item | Value or result |
| --- | --- |
| Reference speed | 100 rad/s |
| Specified rise time | 0.2 s |
| Assumed br | 198 |
| Selected proportional gain K |  |
| Predicted rise time |  |
| Simulated rise time and steady-state error |  |

Table A1. Question 2-1 proportional-controller design.

Calculation, comparison, and discussion:

## 2-2: Physical Implementation and Velocity Estimation

Use the data already collected in the laboratory to compare the physical controller with the simulation and to evaluate both velocity estimates.

- Complete Table A2 for the simulation, discrete velocity estimate, and continuous filtered-derivative estimate.

- Briefly describe the implemented signal scaling, sign conventions, and the two velocity-estimation formulas; place the full Simulink diagram in an appendix.

- Compare rise time, overshoot, steady-state error, and noise or ripple among the three cases.

- Explain discrepancies using friction, sampling, encoder resolution, derivative filtering, saturation, and unmodeled dynamics.

| Method | Rise time | Overshoot | Steady-state error | Noise or ripple | Comments |
| --- | --- | --- | --- | --- | --- |
| Simulation |  |  |  |  |  |
| Discrete velocity estimate |  |  |  |  |  |
| Continuous velocity estimate |  |  |  |  |  |

Table A2. Question 2-2 velocity-estimate comparison.

Comparison and discussion:

## 2-3: PI Velocity Control

Report the final PI design and explain how integral action reveals and counteracts the friction disturbance.

- Complete Table A3 with the final gains, measured rise time, overshoot, steady-state error, and steady-state control effort.

- Compare the P and PI responses using the same velocity-estimation method.

- Explain why the steady-state control effort is nonzero and how it represents the effort required to overcome friction.

- State whether the response satisfied the specified rise-time, overshoot, and steady-state-error requirements.

| PI design item | Value or result |
| --- | --- |
| Kp |  |
| Ki |  |
| Measured rise time |  |
| Measured overshoot |  |
| Steady-state error and control effort |  |

Table A3. Question 2-3 PI-controller tuning.

PI-control analysis and discussion:

## 2-4: Friction Identification

Use the recorded steady-state velocity and control-effort data to identify the asymmetric friction model.

- Complete Table A4 with every positive- and negative-direction measurement used in the fit.

- Include a plot of steady-state control effort versus velocity with separate fitted lines for each direction.

- Complete Table A5 with the Coulomb and viscous coefficients, fitted equations, and a fit-quality measure.

- Write the final piecewise expression for F(ω) and discuss asymmetry, outliers, uncertainty, and units.

| Commanded velocity (rad/s) | Measured velocity (rad/s) | Steady-state control effort | Included in fit? |
| --- | --- | --- | --- |
| 150 |  |  |  |
| 100 |  |  |  |
| 75 |  |  |  |
| 50 |  |  |  |
| 25 |  |  |  |
| -25 |  |  |  |
| -50 |  |  |  |
| -75 |  |  |  |
| -100 |  |  |  |
| -150 |  |  |  |

Table A4. Question 2-4 friction-identification data.

| Direction | Coulomb friction c | Viscous friction b | Fitted expression F(ω) |
| --- | --- | --- | --- |
| Positive |  |  |  |
| Negative |  |  |  |
| R² or fit quality |  |  |  |
| Final values used |  |  |  |

Table A5. Question 2-4 identified friction model.

Friction-model analysis and discussion:

## 2-5: Friction Compensation

Report the friction-compensation results using the values identified in Question 2-4 and the final tuned values from the laboratory.

- Complete Table A6 with the identified values, tuned values, and the reason for each adjustment.

- State the predicted and observed behavior when friction compensation was applied without feedback control.

- Compare P control and PI control with friction compensation, including response speed, steady-state error, and sensitivity to added damping.

- Explain whether integral action remained necessary after compensation and why the compensation was imperfect.

| Friction parameter | Initial Question 2-4 value | Tuned Question 2-5 value | Reason for change |
| --- | --- | --- | --- |
| c+ |  |  |  |
| b+ |  |  |  |
| c- |  |  |  |
| b- |  |  |  |

Table A6. Question 2-5 friction-compensation tuning.

Friction compensation without feedback: prediction and observed behavior:

P-versus-PI comparison with friction compensation:

## 3-1: RWP Parameter Calculations

Calculate the combined physical parameters required by the RWP model.

- Complete Table A7 and show every substitution with units.

- Calculate the total mass, equivalent center-of-mass distance, inertia, and the two natural-frequency quantities.

- Check dimensional consistency and retain enough significant figures for later controller design.

- Explain how the rotor mass and rotor inertia contribute differently to the model.

| Quantity | Given or calculated value | Units | Calculation or source |
| --- | --- | --- | --- |
| mp | 0.2164 | kg | Given |
| mr | 0.0850 | kg | Given |
| Jp | 2.233×10^-4 | kg·m² | Given |
| Jr | 2.495×10^-5 | kg·m² | Given |
| lp | 0.1173 | m | Given |
| lr | 0.1270 | m | Given |
| J |  | kg·m² |  |
| m and l |  | kg and m |  |
| ωnp and ωnp′ |  | rad/s |  |

Table A7. Question 3-1 RWP parameter calculations.

Parameter calculations and interpretation:

## 3-2: Free-Swing Frequency Identification

Report the free-swing frequency obtained from the laboratory data and compare it with the theoretical values.

- Complete Table A8 with the theoretical and measured frequencies and percent differences.

- Explain how the oscillation period or frequency was extracted from the recorded pendulum-angle data.

- Compare ω_np, ω_np′, and the measured value, then identify the value used in the later model.

- Discuss what the slow decay of pendulum oscillations implies about pendulum friction relative to rotor friction.

| Frequency quantity | Value | Comparison or percent difference |
| --- | --- | --- |
| Theoretical ωnp |  |  |
| Theoretical ωnp′ |  |  |
| Measured ωnp′meas |  |  |
| Selected identified ωnp |  |  |

Table A8. Question 3-2 harmonic-frequency comparison.

Frequency comparison and friction interpretation:

## 3-3: Identification of bp and br

Report the polynomial-fit identification of the remaining RWP coefficients.

- Complete Table A9 with the selected data interval, identified coefficients, units, and percent differences.

- Include the original position data and cubic fit, and show the velocity and acceleration obtained analytically from the fit.

- Write the equations used to solve for b_p and b_r after substituting the fitted derivatives and friction model.

- Explain why direct numerical differentiation was rejected and discuss sensitivity to the selected fitting interval.

| Identification item | Value | Units | Notes |
| --- | --- | --- | --- |
| Selected data interval |  | s |  |
| bp |  |  |  |
| br |  |  |  |
| Percent difference from reference |  | % |  |
| Values used in later parts |  |  |  |

Table A9. Question 3-3 identified coefficients.

Identification method, uncertainty, and discussion:

## 4-1: Linearization and Controllability

Present the inverted-equilibrium linearization and establish whether the system is controllable from the motor input.

- Complete the A, B, C, and D matrices using the defined delta-state vector.

- Show the linearization about θ_p = π and identify the equilibrium values.

- Report the open-loop poles and interpret the unstable and zero modes.

- Show the controllability matrix or its rank and explain the result.

State vector x = [δθp, δθ̇p, δθr, δθ̇r]^T

A =

| Column 1 | Column 2 | Column 3 | Column 4 |
| --- | --- | --- | --- |

| [Enter] | [Enter] | [Enter] | [Enter] |
| --- | --- | --- | --- |
| [Enter] | [Enter] | [Enter] | [Enter] |
| [Enter] | [Enter] | [Enter] | [Enter] |
| [Enter] | [Enter] | [Enter] | [Enter] |

B =

Entry

[Enter]

[Enter]

[Enter]

[Enter]

C =

| Column 1 | Column 2 | Column 3 | Column 4 |
| --- | --- | --- | --- |

| [Enter] | [Enter] | [Enter] | [Enter] |
| --- | --- | --- | --- |
| [Enter] | [Enter] | [Enter] | [Enter] |

D =

Entry

[Enter]

[Enter]

Open-loop poles or eigenvalues: [Enter]

Controllability matrix rank: [Enter] of 4

Linearization, controllability, and pole interpretation:

## 4-2: Two-State Feedback Design and Simulation

Report the two-state feedback design and its simulated robustness limits.

- Complete Table A10 with the desired poles, gains, and response observations.

- Complete the two-state column of Table A12, testing δθ_p(0) and δθ̇_p(0) separately, followed by pulse and constant disturbances.

- Show how the selected poles satisfy the stated design constraints and respect input saturation.

- Discuss recovery speed, maximum recoverable initial conditions, disturbance response, and the rotor-speed drift under constant disturbance.

| Two-state design item | Value | Comments |
| --- | --- | --- |
| Desired poles |  |  |
| K gains |  |  |
| Maximum θp initial deviation |  |  |
| Maximum θ̇p initial deviation |  |  |
| Rotor-speed behavior under constant disturbance |  |  |

Table A10. Question 4-2 two-state feedback results.

Two-state feedback analysis:

## 4-3: Three-State Feedback Design and Simulation

Report the three-state feedback design and compare it with the two-state controller.

- Complete Table A11 with the desired poles, gains, retained zero pole, and simulation summary.

- Complete the three-state column of Table A12 using the same separate initial-condition and disturbance tests.

- Explain how rotor-velocity feedback moves the problematic mode and changes constant-disturbance behavior.

- Compare robustness, response speed, control effort, and rotor-speed behavior with Question 4-2.

Provide the required mathematical proof that the linearized, frictionless three-state closed-loop system stabilizes the inverted equilibrium. Substitute the implemented feedback law into the reduced dynamics, show that the relevant equilibrium is the only equilibrium for the controlled states, and verify that the closed-loop poles/eigenvalues are stable.

Conclude explicitly that the controlled states δθp, θ̇p, and θ̇r converge to zero, so θp(t) approaches π for the linearized model.

| Three-state design item | Value | Comments |
| --- | --- | --- |
| Desired closed-loop poles |  |  |
| K gains |  |  |
| Remaining zero pole |  |  |
| Simulation summary |  |  |

Table A11. Question 4-3 three-state feedback design.

Initial-condition testing rule: perturb only one initial state at a time. First vary δθp(0) while δθ̇p(0) = 0; then vary δθ̇p(0) while δθp(0) = 0. Record the maximum recoverable magnitude for each controller in the separate rows below.

| Test condition | Perturbed state or input | Two-State Feedback (4.2) | Three-State Feedback (4.3) | Observer (5.1) |
| --- | --- | --- | --- | --- |
| ① Maximum IC deviation | Pendulum-angle IC, δθp(0), with δθ̇p(0) = 0 (rad) |  |  |  |
| ① Maximum IC deviation | Pendulum angular-velocity IC, δθ̇p(0), with δθp(0) = 0 (rad/s) |  |  |  |
| ② Maximum pulse | Pendulum-arm pulse amplitude, τp |  |  |  |
| ③ Maximum constant disturbance | Constant pendulum-arm disturbance, τp |  |  |  |

Table A12. Integrated robustness comparison, including separate δθp(0) and δθ̇p(0) initial-condition limits.

Two-state versus three-state comparison:

## 4-4: Experimental Three-State Feedback Results

Summarize the physical implementation of the final three-state controller; do not repeat the bench setup procedure.

- Report the final implemented gains and velocity-estimation method.

- Describe any changes made from the simulation design and explain why they were needed.

- Report balance quality, rotor-speed behavior, control saturation, and observed limit-cycle behavior.

- State the result of the required demonstration and reference the supporting plot or video frame when available.

Experimental implementation results and discussion:

## 4-5: Three-State Feedback with Friction Compensation

Compare the physical three-state controller with and without friction compensation.

- Complete Table A13 using observations from the two experimental cases.

- Compare balance quality, limit-cycle amplitude, rotor speed, control effort, and disturbance recovery.

- Explain which changes are consistent with the friction model and which indicate modeling or implementation error.

- State whether compensation improved the controller overall.

| Observation | Without friction compensation | With friction compensation |
| --- | --- | --- |
| Balance quality or limit cycle |  |  |
| Rotor speed and control effort |  |  |
| Disturbance recovery |  |  |

Table A13. Question 4-5 friction-compensation comparison.

Friction-compensation comparison and discussion:

## 5-1: Full-Order Observer Design

Present the full-order observer design used with the three-state feedback controller.

- Complete Table A14 with the controller poles, selected observer poles, and verified observer eigenvalues.

- Fill the Section 5.1 L matrix and state the C matrix used by the observer.

- Show the place calculation conceptually and verify eig(A - LC).

- Justify the observer speed relative to the controller poles and discuss the trade-off with noise and model sensitivity.

Explain why an observer is used in this project and why the four-state observer can be decoupled into two two-state observers; state the practical advantage of the decoupled design.

Prove observer convergence by defining e = x − x̂, deriving the autonomous error dynamics, identifying the equilibrium e = 0, and verifying that the observer-error poles are stable so e(t) approaches zero.

| Observer design item | Value | Comments |
| --- | --- | --- |
| Closed-loop controller poles |  |  |
| Selected observer poles |  |  |
| Verified eig(A - LC) |  |  |

Table A14. Question 5-1 observer pole selection.

L (Section 5.1) =

| Column 1 | Column 2 |
| --- | --- |

| [Enter] | [Enter] |
| --- | --- |
| [Enter] | [Enter] |
| [Enter] | [Enter] |
| [Enter] | [Enter] |

Observer design derivation and justification:

## 5-2: Observer Simulation and Robustness

Evaluate the full-order observer in simulation and compare it with direct three-state feedback.

- Complete the observer column of Table A12 for both separate initial-condition tests, the pulse test, and the constant-disturbance test.

- Compare the observer-based response with the three-state controller using exact or directly estimated states.

- Report the effects of varying model parameters and identify the most influential mismatches.

- Explain why the Section 5.1 observer is highly sensitive despite stable observer poles.

Observer robustness comparison using Table A12:

Model-sensitivity analysis:

## 5-3: Decoupled Observer Redesign

Compare the original observer gain with the block-diagonal redesign and the zeroed-off-block alternative.

- Fill both small L matrices, the combined L matrix, and the Section 5.1 matrix with off-block terms removed.

- Complete Table A15 with eigenvalues, model sensitivity, and simulation results for all three designs.

- Explain how the block-diagonal system structure motivates the redesigned observer.

- Conclude whether a complete redesign is necessary or zeroing the off-block terms is sufficient.

Lsmall,1 =

Entry

[Enter]

[Enter]

Lsmall,2 =

Entry

[Enter]

[Enter]

Lcombined =

| Column 1 | Column 2 |
| --- | --- |

| [Enter] | [Enter] |
| --- | --- |
| [Enter] | [Enter] |
| [Enter] | [Enter] |
| [Enter] | [Enter] |

L5-a with off-block-diagonal terms zeroed =

| Column 1 | Column 2 |
| --- | --- |

| [Enter] |  | [Enter] |  |
| --- | --- | --- | --- |
| [Enter] |  | [Enter] |  |
| [Enter] |  | [Enter] |  |
| [Enter] |  | [Enter] |  |
| Observer version | Eigenvalues | Sensitivity to model changes | Simulation result |
| Original Section 5.1 L |  |  |  |
| Redesigned Section 5.2 L |  |  |  |
| Section 5.1 L with off-block terms zeroed |  |  |  |

Table A15. Question 5-3 observer redesign comparison.

Observer redesign comparison and conclusion:

## 5-4: Observer Implementation on the RWP

Report the experimental observer-based controller and compare it with direct three-state feedback.

- State which L matrix was implemented and report the final controller and observer settings.

- Describe balance quality, noise sensitivity, robustness, control effort, and tuning difficulty.

- Compare observer-estimated velocities with direct velocity estimates or measured behavior.

- Explain why direct measurements or estimates may be more robust than a model-based observer on this apparatus.

Experimental observer results and comparison:

## 6-1: Up and Down Stabilizing Control

Report the required switching-control design and the results used for the Part VI demonstration. This section supports the required 15-point Part VI demonstration.

- State the up and down controller gains and the switching condition based on θ_p.

- Include a block diagram and explain how switching transients were managed.

- Report simulation and experimental results and compare settling in the down position with the uncontrolled pendulum.

Part VI results and discussion:

## Report Conclusion — 10 points

Compare the full-state-feedback controller and the observer-based controller using a clearly stated performance standard.

Define the criteria used to judge controller performance, such as robustness to initial conditions and disturbances, balance quality, rotor-speed behavior, control effort, noise sensitivity, or implementation robustness.

State which controller performed better under the chosen criteria and support the conclusion with numerical or experimental evidence reported earlier.

Conclusion:

## 7-1: Swing-Up Control — Extra Credit (+20 points)

If attempted for extra credit, design and implement the nonlinear swing-up controller and report the simulation and experimental results. A successful RWP demonstration must also be shown to the TA.

- Derive the energy error and state the nonlinear control law.

- Explain rotor-speed limits, friction compensation, and the switching condition to the inverted controller.

- Include simulation and experimental results and discuss unsuccessful attempts or tuning limitations.

Show the implemented swing-up controller to the TA on the RWP. TA verification is required for Part VII extra-credit points to be awarded.

Part VII extra-credit derivation, results, and discussion:

# Final Project Grading Rubric

| Section | Maximum | Criterion | Points |
| --- | --- | --- | --- |
| Prelab | 15 | Required stability, controllability, and prelab analysis. | 15 |
| Demo Part II | 30 | Demonstrate the required friction-compensation cases and explain the model, gains, and block diagram. | 30 |
| Demo Part IV | 30 | Demonstrate three-state feedback, required disturbance tests, and the comparison with and without friction compensation. | 30 |
| Demo Part V | 30 | Demonstrate observer-based control and explain the implemented L matrices and the Section 5.1/5.2 observer designs. | 30 |
| Demo Part VI | 15 | Demonstrate the required up/down switching controller in simulation and on the RWP; explain the gains, switching condition, and observed settling behavior. | 15 |
| Report | 130 | Formal group report; see Table C2 for the detailed 130-point report breakdown. | 130 |
| Base Total | 250 | Required work total: Prelab (15) + Demo II (30) + Demo IV (30) + Demo V (30) + Demo VI (15) + Report (130). | 250 |
| Extra Credit Demo Part VII | +20 | Swing-up control. Demonstrate the implemented swing-up controller to the TA on the RWP; also include a concise discussion and the corresponding C2000/Simulink model in the appendix. | +20 |

Table C1. Final project overall grading rubric.

# Detailed 130-Point Report Grading Breakdown

Report format and style: one report per group; Times New Roman 11-point student-entered text; page numbers; a concise main narrative of about 5–7 pages excluding the cover page; equations, figures, and tables labeled/captioned; complete Simulink diagram printouts included. Style can only reduce the report score, with a deduction from 0 to −50 points for poor formal-report quality, excessive length, missing labels/captions, or weak section-to-section flow.

| Report section | Subcriterion | Required evidence | Points |
| --- | --- | --- | --- |
| Introduction | Sensors | Identify the sensors used and state how many are used. | 3 |
| Introduction | Actuators | Identify the actuator(s) used and state how many are used. | 3 |
| Introduction | Equilibria | Identify the equilibrium positions and state which are stable and unstable. | 3 |
| Introduction | Implementation program | State the program/environment used to implement the controllers. | 1 |
| Mathematical Model | Lagrangian derivation | Derive and explain the differential equations from the Lagrangian. | 5 |
| Mathematical Model | State-space linearization | Linearize the model into the state-space form used for controller design. | 5 |
| Full State Feedback with Friction Compensation | Controller development and friction compensation | Explain the two-state and three-state controller development, how the friction-compensator values were obtained, and whether friction compensation benefited the three-state controller. | 10 |
| Full State Feedback with Friction Compensation | Closed-loop stability proof | Prove the linearized frictionless three-state closed-loop system is stable at the inverted equilibrium: identify the relevant equilibrium and verify stability of the reduced closed-loop dynamics. | 20 |
| Full State Feedback with Friction Compensation | Simulation robustness | Report the maximum recoverable initial-condition deviation, pulse disturbance, and constant perturbation from Simulink. | 10 |
| Full State Feedback with Friction Compensation | Experimental behavior | Describe the behavior observed on the C2000 DAQBoard/RWP implementation. | 10 |
| Full State Feedback with Decoupled Observer | Observer purpose and decoupling | Explain why observers are used, why the four-state observer can be decoupled into two two-state observers, and the advantage of doing so. | 10 |
| Full State Feedback with Decoupled Observer | Observer convergence proof | Define e = x − x̂, derive the observer-error dynamics, and prove the estimated states converge to the real states by verifying the error-system equilibrium and poles. | 20 |
| Full State Feedback with Decoupled Observer | Simulation robustness | Report the maximum recoverable initial-condition deviation, pulse disturbance, and constant perturbation from Simulink. | 10 |
| Full State Feedback with Decoupled Observer | Experimental behavior | Describe the behavior observed on the C2000 DAQBoard/RWP implementation with the observer. | 10 |
| Conclusion | Controller comparison | Define the standard used to judge controller performance and conclude which controller performed better using reported evidence. | 10 |
| Report subtotal |  | Introduction (10) + Mathematical Model (10) + Full State Feedback (50) + Observer (50) + Conclusion (10). | 130 |
| Style deduction | Formal-report quality | Maximum adjustment is 0; up to 50 points may be deducted for poor flow, missing labels/captions, excessive length, or failure to follow the posted formal-report style requirements. | 0 to −50 |

Table C2. Detailed 130-point final-project report rubric and style deduction.

Part VI is required and carries 15 demonstration points within the 250-point base project grade. The Section 6 write-up is required supporting documentation for that demonstration and does not create an additional report subscore beyond the 130-point report rubric. Part VII is the only extra-credit section and is worth up to +20 points. If attempted, the swing-up controller must be demonstrated to the TA, and the submission must include a concise discussion of the approach and the corresponding C2000 DAQBoard/Simulink model(s) in an appendix.