# Lab 1: Simulation Using the Analog Computer

Report by: [Enter your name]

Lab partner: [Enter partner name or “None”]

Lab TA: [Enter TA name]

Section: [Enter section]

## Question 1: Theoretical and experimental response results (15 points)

### Theoretical and experimental results (5 points)

Experimental percent overshoot is calculated using:

Mp=maximum value−steady-state valuesteady-state value×100%

Use the following report equations for the theoretical values. For this laboratory, normalized time allows ωₙ = 1 rad/s.

Mp=exp−πζ1−ζ2,0<ζ<1,0,ζ≥1.

tr≈1.2−0.45ζ+2.6ζ2ωn,0<ζ<1.2,4.7ζ−1.2ωn,1.2≤ζ<3.

ts≈−0.5ζωnln1−ζ2400,0<ζ≤0.69,6.6ζ−1.6ωn,0.69<ζ<3.

| Damping ratio, ζ | Mₚ theoretical (%) | Mₚ experimental (%) | tᵣ theoretical (s) | tᵣ experimental (s) | tₛ theoretical (s) | tₛ experimental (s) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.0 |  |  |  |  |  |  |
| 1.5 |  |  |  |  |  |  |
| 1.0 |  |  |  |  |  |  |
| 0.8 |  |  |  |  |  |  |
| 0.7 |  |  |  |  |  |  |
| 0.5 |  |  |  |  |  |  |
| 0.3 |  |  |  |  |  |  |
| 0.2 |  |  |  |  |  |  |

Table 1. Theoretical and experimental response metrics for each damping ratio.

Include one sample response plot with the points used to determine percent overshoot, rise time, and settling time clearly marked.

Attach the annotated sample plot here. Add alternative text that describes the axes, damping ratio, response curve, marked points, and the values represented by those points.

### Comparison of theoretical and experimental results (5 points)

Compare the computed theoretical values with the experimental values. Discuss whether the equations in the Lab 1 report section match the measured responses and explain meaningful differences.

Put Discussion Here

### Variation with damping ratio ζ (5 points)

Discuss how percent overshoot (Mₚ), rise time (tᵣ), and settling time (tₛ) vary as ζ changes.

Put Discussion Here

## Question 2: Damping ratio, pole locations, and response metrics (15 points)

### Effect of ζ on pole locations (5 points)

Derive the two pole locations in terms of ζ; you may assume ωₙ = 1. Include either a sketch or graph of the pole locations as ζ increases, or a clear written description of that graph.

Begin with the characteristic equation:

s2+2ζωns+ωn2=0

Put Discussion Here

### Effect of pole locations on an underdamped system (5 points)

State the range of ζ for an underdamped system. Explain how the poles move as ζ increases and how that movement affects Mₚ, tₛ, and tᵣ. Address the two different effects that pole motion can have on settling time.

Put Discussion Here

### Effect of pole locations on overdamped and critically damped systems (5 points)

State the ranges or values of ζ for overdamped and critically damped systems. Explain how the poles move as ζ increases and how that movement affects Mₚ, tₛ, and tᵣ.

Put Discussion Here

## Question 3: First-order approximation of an overdamped system (10 points)

Investigate the approximation of an overdamped second-order system by retaining only the pole closer to the origin, pmin.

H1s=p1p2s+p1s+p2 ⇒ H2s=pmins+pmin

### Similarities and differences between the second-order systems and first-order approximations (6 points)

Plot the second-order step responses and their first-order approximations for ζ = 1.5, ζ = 5, and ζ = 40, with ωₙ = 1. Explain how the approximate responses are similar to and different from the original second-order responses.

Put Discussion Here

### Effect of the magnitude of ζ on approximation accuracy (4 points)

Explain how the value of ζ affects the accuracy of the first-order approximation.

Put Discussion Here

## Required attachments

Plots obtained during the laboratory session.

One sample response with the points used to calculate Mₚ, tᵣ, and tₛ marked.

Step-response plots comparing the second-order systems with their first-order approximations for ζ = 1.5, 5, and 40.

# Lab 1 grading rubric

| Section | Section maximum | Subsection | Criterion | Criterion points |
| --- | --- | --- | --- | --- |
| Prelab | 20 | a. Differential equation | Write the differential equation for the given transfer function. | 3 |
| Prelab | 20 | b. Normalized-time conversion | Using τ/ωₙ = t, express dy/dt in terms of dy/dτ. | 1 |
| Prelab | 20 | b. Normalized-time conversion | Using τ/ωₙ = t, express d²y/dt² in terms of d²y/dτ². | 2 |
| Prelab | 20 | b. Normalized-time conversion | Write the normalized differential equation. | 3 |
| Prelab | 20 | c. Highest derivative | Isolate d²y(τ)/dτ². | 2 |
| Prelab | 20 | c. Highest derivative | Integrate to express y(τ). | 1 |
| Prelab | 20 | d. Block diagram | Create the all-integrator block diagram. | 3 |
| Prelab | 20 | e. Patch panel | Complete the analog-computer patch-panel wiring diagram. | 5 |
| Report | 40 | Question 1 | Complete the theoretical/experimental results and include one annotated sample response. | 5 |
| Report | 40 | Question 1 | Compare the theoretical and experimental results. | 5 |
| Report | 40 | Question 1 | Discuss the variation of Mₚ, tₛ, and tᵣ with ζ. | 5 |
| Report | 40 | Question 2 | Derive and explain the effect of ζ on pole locations. | 5 |
| Report | 40 | Question 2 | Explain the effect of pole locations on Mₚ, tₛ, and tᵣ for an underdamped system. | 5 |
| Report | 40 | Question 2 | Explain the effects for overdamped and critically damped systems. | 5 |
| Report | 40 | Question 3 | Explain similarities and differences between the second-order systems and first-order approximations. | 6 |
| Report | 40 | Question 3 | Explain how the magnitude of ζ affects approximation accuracy. | 4 |
| TA Check | 10 | Required TA Checks | Complete all required TA Checks. Show requested plots, data, wiring, code, or block diagrams and answer TA questions. | 10 |
| Lab Total | 70 | Prelab + TA Check + Report | Prelab (20) + TA Check (10) + Report (40) = 70. | 70 |

Table 2. Lab 1 rubric organized by major section, subsection, criterion, and criterion points.