ECE 576 Homework 8

Should Be Done Before the Final

1.	Using the WSCC_Wind_Type2_TwoMass PowerWorld case on Lecture 24, create the torque-speed curves for the Type 2 generator at bus 3 for the Rext = Rmin and for Rext=Rmax

2.	Using the WSCC_Wind_Type3 PowerWorld case, for the Type 3 generator at bus 3 derive the initial values for Ip and Eq (labeled EQtMD on the block diagram on Lecture 24, Slide 19; assumed Iqextra is zero).

3.	Book 8.4

4.	As was mentioned in lecture 25, the PowerWorld case B4_GENROU_Sat_SMIB (with the exciter) is unstable when Q4 is 0.  The block diagram for the very simple STAB1 stabilizer is given below.  Starting with the PowerWorld default parameters, determine a good set of parameters for stabilizing this system.

5.	Book 8.8

6.	For the PowerWorld case WSCC_HW8_Prob6 the exciter at generator 2 has been modified to given an unstable response.  Comment on the role the limits and saturation for this exciter have in limiting this response?

7.	Again with the WSCC_HW8_Prob6 case, as the generator 2 exciter Ka value is decreased eventually all the SMIB eigenvalues for this generator become negative.  At what value does this occur?  Be sure to Re-Initialize the case between each SMIB solution with different parameters.  How closely does the response expected from the SMIB match the actual system response?  Comment on why they might be different.